Name: ZOHAIB HASSAN SOOMRO

RollNo#: 19SW42

Subject:

User exe mode

Privilege exe mode :

Type en or enable

Global Config mode:

type config t/configure terminal

Line Mode:

Now we can do any configurations

Going back to privilege exe mode: ctrl+Z

Going back to user exe mode: disable

Rename a switch(being in global config mode):

Hostname switchNameHere

Secure a switch:

Set a password to secure a switch while user goes into privileges exe mode.

Type: enable secret passwordHere

Telnet command to access a switch by its ip.

Set /Configure an IP for switch:

Type this:

- Interface vlan 1

- Ip address ipaddressHere(e.g 192.168.0.9) subNetMaskHere(255.255.255.0)

Line mode to global config mode:

Type exit

Set gateway now:

Type ip default-gateway gatewayHere (e.g192.168.0.254)

If we want to put password when someone wants to access our switch using telnet command(while in privilege exe mode).

- Every networking device has capacity to hold telnet like here we have 0 to 15

- line vty 0 15

- password muet

- login

Show version command:

Show running-config

Enrypt the telnet password to increase security:

Console cable ->Rs 232 on pc side and console on switch side